| Prepared by: Tina Peng | Approved by: Thomas Sun | Date of Issue: 2009/3/18 Date of Revise: |
| --- | --- | --- |

History of Revision

| Version | Date | Page | Description | Remark |
| --- | --- | --- | --- | --- |
| 1.0 | 2008/10/21 | All | 1st official Release |  |
| 1.1 | 2008/11/28 | All | 1. Add "Previous". "Play/Pause". "Next". "Stop". "Fast Forward". "Rewind" icon. 2. Modify "Silent Mode On/Off". "Caps Locks". "Num Locks". "Scroll Locks". "Mute On/Off". "Volume" icon. 3. Solve 3D OSD icon hang up bug when press IE explorer while icon animation is still running. 4. Solve 3D OSD icon hang up bug when hold down any key continuously. 5. Remove OSD beep voice function. 6. Modify "Caps Locks On". "Num Locks On". "Scroll Locks On". "Silent Mode On". "Touchpad Off". "WLAN On". "Web Cam On". "BlueTooth On". "Mobile Broadband On" OSD-Bar Icon. |  |
| 1.2 | 2008/12/12 | P1,2,3,4 5,7,8,10 | 1. Solve OSD icon turns into gray tone when drag to task bar. 2. Solve 'Bluetooth icon' hang up bug (will not disappear after showed up). 3. Solve OSD doesn't release system memory (after icon display) issue. 4. Change OSD-Bar Icon. 5. Change "wireless off" icon. |  |
| 0.01 | 2009/1/6 | P1~4,10 | 1. Modify OSD icon. 2. Modify osd.exe icon. 3. Add OSD icon continuously shifts from left to right when point cursor on OSD bar. 4. Add OSD icon rotate effect when activated. | Form now on, OSD promotes Version from 0.01 |
| 0.02 | 2009/1/8 | P1.3.9.10 | 1. Solve "Fast Forward" and "Rewind" don't work problem. |  |
| 0.03 | 2009/1/22 | P1.10 | 1. Solve OSD icon hang up bug when press Media button of MMB while icon animation is still running. 2. When move OSD bar to top right corner of the screen, OSD bar will get outside of the screen. |  |
| 1.00 | 2009/3/6 | P2-5 | 1. Add "Enable External Keyboard" option. 2. Add to support X64 OS. 3. AP and driver need to be installed separately. |  |
| 1.01 | 2009/3/18 | P2-5,8.11 | 1. Solve Live! OSD installation issue under x64 2. Display OSD bar in tool bar only. 3. Solve silent mode issue. 4. Add "Auto-Brightness On/Off" icon.(For T30) 5. Add "Auto-KB Brightness On/Off" icon.(For T30) |  |

OSD Spec. Description

- EC will send out the scan code after user press the button; AP will show the OSD icon accordingly.

- The OSD icon should be synchronized as OS ones.

- OSD AP function and setting

1. OSD-System tray

  - Press “About” show OSD version notice.

  - Press “Quit” Disable all OSD function

  - Press “Enable Num Lock of External Keyboard” selects or deselects 'Enable External Keyboard' to enable or disable external keyboard.

  - Dimension：Pixel (W)20 x (H)20

  - Include “Caps Lock ”, ”Num Lock”, “Scroll Lock”, “Touchpad ”, “Silent”, “Volume”, “Brightness”,”Mute”,” Wireless”,”BlueTooth”,“HSPA”, ”Web Cam”, ”Web Browser”,”Previous”,”Play/Pause”,”Next”,”Stop”,”Fast Forward”,”Reward”.                  (The function depends on the definition of product specification)

  - OSD icon rotates effect when activated.

2. OSD-Bar

  - Dimension：Pixel (W)20 x (H)20

  - Only Touchpad has disable icon; other functions: disable = no icon display.

  - Display OSD bar in tool bar only.

  - OSD Can recollect state when S3&S4,Warmboot,Coldboot

3. OSD-On Screen Display

  - Display time：Fixed value delay 2 seconds.

  - Display location：In the middle of screen

  - Dimension：Pixel(W)200 x (H)200

  - Distribute OSD into classes:”Function-Key”.

4. OSD AP via the port 68/6C access.

5. OSD AP when install and uninstall add reset dialog function.

6. OSD AP play cartoon.

7. OSD AP can choose Everyone item & Just me item as below photo.

- Everyone item provide all account use.

- Just me item provide single account use

- OSD default item on Everyone.

8. When mouse move to the icon at OSD-Bar or TaskBar, the icon can move, for example

→→

9. OSD AP does function of "Fast Forward" and "Rewind".

10. AP and driver need to be installed separately, and it need ECSWMI.dll to install normal.

- Install Live! Control Center (driver) at first.

- Install Live! OSD after.

NOTE: Live! OSD is guaranteed to be compatible with default/built-in devices of this notebook.

Live! OSD cannot guarantee compatibility with other external devices.

OS Support: Windows Vista & Windows XP. **Every OSD version must be tested and verified by related

project’s EC engineer before release to each project team.

Display Outline

TaskBar

Screen Display

Screen Display Icon (The function depends on the definition of product specification)

| T/P On/Off |  |
| --- | --- |
| WebCam On/Off |  |
| Silent Mode On/Off |  |
| Caps Locks |  |
| Num Locks |  |
| Scroll Locks |  |
| WLAN On/Off |  |
| BlueTooth On/Off |  |
| HSPA On/Off |  |
| Brightness |  |
| Mute On/Off |  |
| Volume |  |
| Previous |  |
| Play/Pause |  |
| Next |  |
| Stop |  |
| Fast Forward |  |
| Rewind |  |
| Web Browser |  |
| ALS On/Off |  |
| KB ALS On/Off |  |

OSD-Bar Icon (The function depends on the definition of product specification)

| Caps Locks On |  |
| --- | --- |
| Num Locks On |  |
| Scroll Locks On |  |
| Silent Mode ON |  |
| Touchpad Off |  |
| WLAN On |  |
| Web Cam On |  |
| BlueTooth On |  |
| Mobile Broadband On |  |

Scan Code Definition

| Function | User Define(AMI) | Scan code(Phoenix) | Description |
| --- | --- | --- | --- |
| Silent Mode(On) | 9A | 0xE0 0x1A |  |
| Silent Mode(Off) | A2 | 0xE0 0x22 |  |
| Touchpad (On) | 85 | 0xE0 0x05 |  |
| Touchpad (Off) | 86 | 0xE0 0x06 |  |
| Webcam (On) | 84 | 0xE0 0x04 |  |
| Webcam (Off) | 8C | 0xE0 0x0C |  |
| WLAN (On) | C9 | 0xE0 0x49 |  |
| WLAN (Off) | 8B | 0xE0 0x0B |  |
| Bluetooth (On) | 8A | 0xE0 0x0A |  |
| Bluetooth (Off) | 81 | 0xE0 0x01 |  |
| WLAN_BT(On) | D6 | 0xE0 0x56 |  |
| WLAN_BT(Off) | D7 | 0xE0 0x57 |  |
| HSPA(On) | 89 | 0xE0 0x09 |  |
| HSPA (Off) | FB | 0xE0 0x7B |  |
| Edge (On) | 88 | 0xE0 0x08 |  |
| Edge (Off) | 8F | 0xE0 0x0F |  |
| TV (On) | A9 | 0xE0 0x29 |  |
| TV (Off) | D8 | 0xE0 0x58 |  |
| Num Lock(On) | FF | 0xE0 0x7F | For update Num status use |
| Scroll Lock(On) | E1 | 0xE0 0x61 | For update Scroll status use |
| Caps Lock(On) | 87 | 0xE0 0x07 | For update Caps status use |
| OSD_Volume Up | E2 | 0xE0 0x62 | No Use |
| OSD_Volume Down | E3 | 0xE0 0x63 | No Use |
| OSD_Mute | E4 | 0xE0 0x64 | No Use |
| Brightness Up | B1 | 0xE0 0x31 |  |
| Brightness Down | C1 | 0xE0 0x41 |  |
| OSDDebug | EF | 0xE0 0x6F | OSD debug use |
| Next Track | CD | 0xE0 0x4D | Win key |
| Previous Track | 95 | 0xE0 0x15 | Win key |
| Stop | BB | 0xE0 0x3B | Win key |
| Play/Pause | B4 | 0xE0 0x34 | Win key |
| Mute | A3 | 0xE0 0x23 | Win key |
| Volume Up | B2 | 0xE0 0x32 | Win key |
| Volume Down | A1 | 0xE0 0x21 | Win key |
| Media Player | AA | 0xE0 0x50 | Win key |
| Mail | C8 | 0xE0 0x48 | Win key |
| WWW Search (IE) | 90 | 0xE0 0x10 | Win key |
| OSDBeep | 97 | 0xE0 0x17 |  |
| Update Status | F8 | 0xE0 0x78 |  |
| Fast Forward | F3 | 0xE0 0x73 | OSD AP does the function. |
| Rewind | F9 | 0xE0 0x79 | OSD AP does the function. |
| Auto-Brightness(On) | B9 | 0xE0 0x39 | For T30. |
| Auto-Brightness(Off) | D1 | 0xE0 0x51 | For T30. |
| Auto-KB Brightness(On) | D3 | 0xE0 0x53 | For T30. |
| Auto-KB Brightness(Off) | DC | 0xE0 0x5C | For T30. |

OSD design rule

A. EC RAM address fixed for OSD

| Address | Description |
| --- | --- |
| 0757h | Bit1:TouchPad / Bit2:WUSB / Bit4:Webcam / Bit5:BT / Bit6:HSPA(3G) / Bit7: Wlan |
| 0758h | Bit0:Scroll / Bit1:Num / Bit2:Caps / Bit3: Silent mode |
| 0759h | Bit0: OSD tool ready |

B.OSD Bar/ Screenr icon display rule

| T/P On/Off | Touchpad (On/OFF) scan code |
| --- | --- |
| WebCam On/Off | Webcam (On/OFF) scan code |
| WLAN On/Off | WLAN (On/OFF) scan code |
| HSPA/Edge On/Off | HSPA/Edge (On/OFF) scan code |
| Silent Mode On/Off | Silent Mode(On/OFF) scan code |
| Caps Locks | Check EC RAM address 0758 bit2 when receive Cap Lock(On) scan code |
| Num Locks | Check EC RAM address 0758h bit1 when receive Num Lock(On) scan code |
| Scroll Locks | Check EC RAM address 0758h bit0 when receive Scroll Lock(On) scan code |
| Mute | Mute(On/OFF) scan code |
| Volume Scroll | Check system volume when receive volume up / down scan code (OSD control volume) |
| Brightness Scroll | Check EC RAM address 0443 bit0 –bit2 in XP. Check WMI in Vista. |

C .OSD debug

1. EC send OSDDebug(6Fh) scancode when receive 62h/66h port 90h command.

2. EC send OSDDebug(6Fh) scancode when OSD ready (0759h[0] 01 ).

3. Use AMI kernel muse be send User Define value, then use phoenix kernel muse be send Scan code value.

D. To EC

1. For test convenient, we supply Keyboard.ini (in OSD’s Folder) to modify, every EC can modify contents of Keyboard.ini for requirement.

2 .For avoiding mistakes scan code of "Volume Up"."Volume Down"."Mute" is

| Function | User Define(AMI) | Scan code(Phoenix) | Description |
| --- | --- | --- | --- |
| Mute | A3 | 0xE0 0x23 | Win key |
| Volume Up | B2 | 0xE0 0x32 | Win key |
| Volume Down | A1 | 0xE0 0x21 | Win key |

3. Brightness value 0~7 level.